I. BASIC DATA ON THE PROPOSER

I.1. 	Name:  	The Slovak Road Administration, Bratislava

I.2.       Company registration No:		003 328

I.3.	Registered office:		Miletičova 19, 820 09 Bratislava

I.4. 	Authorized representative:

Ing. Jozef Fabian – director of IV and SC Košice

The Slovak Road Administration, Investment Road Construction and Administration, Košice

Kasárenské námestie č. 4

041 01 Košice

I.5.	Person authorized to provide relevant information on proposed activity:

Ing. Gabriela Mareková

The Slovak Road Administration, Investment Road Construction and Administration, Košice

Kasárenské námestie č. 4

041 01 Košice

Phone no.: 055/727242

Ing. Ján Šťastný

The Slovak Road Administration, Investment Road Construction and Administration, Košice

Kasárenské námestie č. 4

041 01 Košice

jan.stastny@ssc.sk

Phone no.: 051/7582028

Ing. Ján Longa  (Chief solutionist)

DOPRAVOPROJEKT, a.s.

Kominárska 2-4

823 03 Bratislava

longa@dopravoprojekt.sk

Phone no.: 02/502 34 392

II. BASIC DATA ON THE PROPOSED ACTIVITY

II.1. 	Name

I/18 and I/74 Lipníky – Ubľa, relocation of the road

II.2. 	Purpose

Nowadays, a concept of development of the motorways and highways of Slovakia does not assume with location of motorway or highway in this area of interest. Main traffic function in this locality is ensured by the network of roads of Class I. that is connected to the highway R4 in proposed interchange crossing Lipníky, as well as by the roads I/18 near the city of Michalovce and I/79 near the city of Sečovce connected to proposed motorway D1.

Connection of western and eastern parts of Slovakia forms important part of not only domestic, but also international automobile traffic. Quality of this connection markedly influences development speed of all concerned regions. The area of Prešov region, which is being solved, belongs among those regions, where insufficient traffic infrastructure means main obstruction of further economical development.

Nowadays, all traffic is realized on the roads of Class I. In the affected area, a route has got few point defects mainly caused by longitudinal and vertical alignment of the road and uncontrolled crossroads. These point defects are so important that they form critical accident localities in this area.

The importance of above mentioned traffic connection is supported by increasing need to improve accessibility of all regions in Slovakia with the aim to balance existing regional disparities.

According to the act no. 24/2006 Coll. on environmental impact assessment and on modification and amendment of some acts, the aim of environmental examination is to provide information about proposed activity, about state of the environment in the locality, where proposed activity will be realized, about presupposed impacts of proposed activity on environment and about proposals of measures for their exclusion, decrease or compensation.

II.3. 	Character of proposed activity

New construction

II.4. 	Location of proposed activity:

region:			Prešov, Košice

district:			Prešov, Vranov nad Topľou, Michalovce, Humenné, Snina

cadastral zone: 	Nemcovce, Šarišská Poruba, Lipníky, Chmeľov, Pavlovce, Radvanovce, Medzianky, Hanušovce nad Topľou, Petrovce, Bystré, Čierne nad Topľou, Vyšný Žipov, Jastrabie nad Topľou, Hlinné, Vranov nad Topľou, Soľ, Čaklov, Hencovce, Nižný Hrabovec, Strážske, Brekov, Humenné, Lackovce, Jasenov, Hažín nad Cirochou, Kamenica nad Cirochou, Modra nad Cirochou, Dlhé nad Cirochou, Belá nad Cirochou, Snina, Stakčín, Kolonica, Ladomirov, Klenová, Ubľa

II.5. 	Term of commencement and completion of construction and operation of proposed activity:

- term of commencement – 2015

- term of construction completion and its putting into operation - 2020

With respect to adjusted construction procedure of motorways and highways, the above mentioned terms will be specified according to current situation during construction.

In terms of construction extent of road relocation, division to separate parts is recommended as following:

road I/18 in the section Lipníky – Strážske

- construction – I/18 Lipníky – Hanušovce west (ZÚ – km 7.800)

- construction – I/18 Hanušovce west – Bystré (km 7.800 – km 15.000)

- construction – I/18 Bystré – Vranov north (km 15.000 – km 30.800)

- construction – I/18 Vranov north – Vranov south (km 30.800 – km 37.200)

- construction – I/18 Vranov south – Nižný Hrabovec (km 37.200 – km 42.500)

- construction – I/18 Nižný Hrabovec – Strážske south (km 42.500 – 48.260).

road I/74 in the section Strážske – Ubľa

- construction – I/74 Strážske south –Brekov (ZÚ – km 7.400)

- construction – I/74 Brekov – Humenné (km 7.400 – km 13.500)

- construction – I/74 Humenné – Snina west (km 13.500 – km 26.000)

- construction – I/74 Snina west – Snina east (km 26.000 – km 34.000)

- construction – I/74 Snina east – Kolonica (km 34.000 – 42.500)

- construction – I/74 Kolonica – Ubľa (km 42.500 – 57.000).

II.6. 	Brief description of technical and technological solution

In all variants, proposed relocation of the road I/18 will begin westwards from the village Lipníky in interchange crossing “Lipníky”. Southwards from the city Strážske, the road I/18 is connected in interchange crossing “Strážske“ - south" to the road I/74, while the road I/18 is from this interchange crossing led southwards in the direction to Michalovce as a part of other construction.

The beginning of relocation of the road I/74 is situated in interchange crossing “Strážske - south“ and its end is in existing border crossing Ubľa - Malyj Breznyj on Slovak - Ukrainian border.

Solving of direction and inclination rations was influenced by configuration of the terrain, geological conditions of the locality and mainly be location of built-up areas.

Proposed speed of relocation of the road of category C 11,5 is 100 km/hour with horizontal curve radiuses from 500 m to 2,500 m. In the valley of rivers Topľa, Laborec and Cirocha, spatial alignment of the road relocation is equivalent to proposed speed 120 km/hour (C 11,5/120). Sections of road relocation constructed in the category C 11,5/100 (120) will be completed as four-way road of the category R 24,5/100 (120), while proposed road relocation forms right half of this forward-looking statement category. With respect to forward-looking statement of proposed road relocation to the network of highways and limited access of the vehicles prospective highway according to act on traffic on ground communications for vehicles with construction speed lower than 40 km/hour, this communication is situated mainly outside of built-in area or at the edge of higher urban localities.

Proposal of variant solutions of road relocation

Section of road relocation I/18 Lipníky – Strážske

From the beginning of the section, after 4.000 km, the section goes through hilly up to mountain area of the peak „Petič“ with elevation of 340 meters. In this section, relocation of the road I/18 is situated at the edge of built-in locality of the village Lipníky. Immediately under the saddleback of the mount Petič, the road relocation follows the border of districts Prešov and Vranov Topľou. From the saddleback of the mount Petič, it goes through the valley of Medziansky potok (Medziansky stream) through built-in area of the village Pavlovce, part Podlipníky. This road goes through the valley of Medziansky potok (Medziansky stream) up to built-in area of the city Hanušovce nad Topľou, while this section of the road passes undulating and hilly terrain. Versions B blue I/18 and C green I/18 are in the section of km 3.000 up to 8.000 led through route of the railway no. 193 Humenné - Prešov, while version B blue is situated directly alongside the railway through built-in area of the city Hanušovce nad Topľou. In the section of km 12.000 up to 14.000, the road relocation - version A red I/18 passes the hill area in the corridor of railway no. 193 Humenné - Prešov and it intersects built-in area of the village Bystré in gap site. In km 15.000, it crosses the railway Humenné - Prešov. In the section from km 16.000 up to km 29.000, the relocation of the road is led directly through plain area of the river Topľa with its multiple crossing. The road relocation passes the areas of the villages Čierne nad Topľou, Vyšný Žipov, Hlinné, Jastrabie nad Topľou, Čaklov and Vranov nad Topľou.

The route of road relocation of the version B is in the section km 31.000 u to 38.000 designed in plain terrain of flood plain of the river Topľa; the crosses the river several times, as well as its side feeders. Version B blue I/18 is led through built-in area of the city Vranov nad Topľou in its narrowest point near the river Topľa. In the south part of the city, the road relocation is situated between the river Topľa, which is crossed by the road several times, and industrial park Ferovo. In the whole route, the road relocation crosses several erosive grooves with permanent or temporary streams and it also crosses existing railway no. 192 Trebišov - Vranov nad Topľou. To the south of industrial park Ferovo, it is led through the locality used for agricultural purposes. Southwards from the city Vranov nad Topľou, it is connected to existing roads I/15 and I/18 in the place of crossing the road I/18 with the railway no. 193 Humenné - Prešov.

Versions A red I/18 and C green I/18 of the road relocation are located westwards from the city Vranov nad Topľou, part Čemerné on slightly undulating terrain and are led directly through the locality of protected bearing locality of brick clay above the part Čemerné. Southwards from the city district Čemerné, the road relocation crosses existing road I/79, where interchange crossroad of the road relocation I/18 with the road I/79 is situated. The road relocation passes the valley of the river Topľa, it crosses the river and railway no. 192 Trebišov - Vranov nad Topľou by bridge objects, behind the river Topľa, the road relocation is led through agriculturally used locality. Southwards from the city Vranov nad Topľou, the road relocation is connected to existing roads I/15 and I/18 in the place of crossing of the road I/18 with the railway no. 193 Trebišov - Humenné.

In the section of km 38.000 up to 44.000, the road relocation passes goes through the corridor of railway Humenné - Prešov near built-in area of the village Nižný Hrabovec. Between the village Nižný Hrabovec and the city Strážske, the road relocation passes the hill area and southwards from the city Strážske, it is led in the corridor of railways Humenné - Prešov and Strážske - Michalovce.

Section of road relocation I/74 Strážske – Ubľa

The beginning of relocation of the road I/74 in the crossroad Strážske south in the corridor of the railway Strážske - Michalovce. Relocation of the road I/74 in the section of built-in area up to km 8.000 is led parallel with this railway and so its forms common traffic corridor. From the terrain morphology point of view, the road relocation I/74 passes the plain terrain of flood plain of the river Laborec from the beginning of this section up to km 15.000. In km 1.000, it enters into built-in area of the city Strážske and railway station. Northwards from the city Strážske up to south edge of built-in area of the city Humenné, it passes plain territory in the corridor of railway Humenné - Prešov. In this section, it goes eastwards from the village Brekov between the village and area of agricultural cooperative. In km 9.000, the road relocation enters into built-in area of the city Humenné. Version B blue I/74 is situated in plain area in parallel with the river Laborec. Version A red I/74 is situated in slightly undulating territory eastwards from built-in area of the city Humenné.

Northwards from the city Humenné up to the village Belá nad Cirochou, it passes plain terrain of flood plain of the river Cirocha, which is crossed by this road several times. Version B blue I/74 is situated eastwards from the built-in area of the villages Modra nad Cirochou and Dlhá nad Cirochoou in slightly undulating or hilly locality. Northwards from the village Belá nad Cirochou, the road relocation passes east edge of the built-in area of the city Snina. In this section, the road relocation is led through mountains locality with few deep valleys of the rivers with steep slopes. Northwards from the city Snina, it passes plain territory in parallel with the river Cirocha up to the village Stakčín. Southwards from the village Stakčín, relocation of the road I/74 is led from the valley of the river Cirocha eastwards and is directed to the village Ubľa, while in this section (km 36.000 up to its end) it passes hilly or mountain terrains with tendencies to land-slide. In this section, the road relocation passes the villages Kolonica, Ladomirov and Ubľa. Eastwards from the village Ubľa up to the state border of Slovak republic with Ukraine, the road relocation is led through the valley of Ublianka stream. The end of relocation of the road I/74 is situated in the place of existing border crossing Ubľa/Malyj Bereznyj.

Subversion A greenI/74

The design of subversion A green I/74 meets the requirements of Ministry of Environment of the Slovak Republic. Part of the relocation of the road I/74 subversion A green is up to km 41,500 identical with version A red. The subversion A green is being diverted from version A red in km 41,500 and is connected to a bridge in km 42,000, which crosses a valley with Kolonička creek. Subversion A than continues with embankment to a bridge in km 42,500, which crosses a valley with an unnamed creek and the existing road I/74. MÚK “Kolonica” is connected into this part of the road. Relocation of the road I/74 continues with embankment to bridge located in km 43,400, which crosses a valley with Malý Tapovec creel. The route than changes to slopes situated east from village Kolonica and with cuts it goes into the valley of Poľana creek. There it crosses erosion scores with temporary and permanent influx of creek Poľana. In km 48,000 the subversion A green is connected to an existing road I/74 respectively version A red MÚK “Ladomirov east”. From km 48,000 till the end of the route, it is identical with the version A red.

New solutions of crossroads

With respect to forward-looking statement of proposed roads to the network of highways of SR, there are several, mostly interchange crossings (MÚK) proposed for the relocation of roads I/18 and I/74. Level crossings are proposed only in case of blue version, which uses area of existing road in the section through built-in area of the village Nižný Hrabovec. This version is not recommended also because of forward-looking statements.

Version A red I/18

- km 0.500 MÚK “Lipníky”

- km 4.000 MÚK “Petič”

- km 6.000 MÚK “Podlipníky”

- km 7.800 MÚK “Hanušovce west”

- km 15.000 MÚK “Bystré”

- km 30.800 MÚK “Vranov north”

- km 34.500 MÚK “Čemerné”

- km 37.200 MÚK “Vranov south”

- km 41.400 MÚK “Nižný Hrabovec”

- km 47.600 t MÚK “Strážske south”

Version A red I/74

- km 1.000 MÚK “Strážske centre”

- km 2.800 MÚK “Brekov”

- km 7.400 MÚK “Humenné west”

- km 10.500 MÚK “Jasenov”

- km 13.500 MÚK “Humenné east”

- km 21.500 MÚK “Modra nad Cirochou”

- km 29.000 MÚK “Snina west”

- km 34.000 MÚK “Snina east”

- km 36.000 MÚK “Stakčín”

- km 42.500 MÚK “Kolonica”

- km 45.200 MÚK “Ladomirov west”

- km 48.000 MÚK “Ladomirov east”

- km 51.500 MÚK “Ubľa”

- km 56.500 MÚK “Ubľa border”

Version B blue I/18

- km 0.500 MÚK “Lipníky”

- km 11.500 MÚK “Hanušovce east”

- km 20.000 MÚK “Hlinné”

- km 30.200 MÚK “Vranov north”

- km 34.000 MÚK “Vranov centre”

- km 35.500 MÚK “Vranov south”

- km 38.400 level crossing with existing road and road II/554

- km 39.550 existing level crossing with the road II/554

- km 45.700 MÚK “Strážske south”

Version B blue I/74

- km 1.000 MÚK “Strážske centre”

- km 2.700 MÚK “Brekov”

- km 7.400 MÚK “Humenné west”

- km 11.050 “Humenné centre”

- km 13.100 MÚK “Humenné east”

- km 26.000 MÚK “Belá nad Cirochou”

- km 33.500 MÚK “Snina east”

- km 36.000 MÚK “Stakčín”

- km 42.500 MÚK “Kolonica”

- km 53,000 MÚK “Ubľa”

- km 57.500 MÚK “Ubľa border”

Version V green I/18

- km 0.500 MÚK “Lipníky”

- km 9.700 MÚK “Hanušovce”

- km 12.700 MÚK “Hanušovce east”

- km 27.200 MÚK “Soľ”

- km 30.800 MÚK “Vranov north”

- km 34.500 MÚK “Čemerné”

- km 37.200 MÚK “Vranov south”

- km 41.400 MÚK “Nižný Hrabovec”

- km 47.600 MÚK “Strážske south”.

Subversion A green I/74

- all of the crossroads are identical with the version A red, except of MÚK “Ladomirov west”

NOTICE: MÚK - interchange

Proposal of relocations solution (modifications, reconstructions) of the roads of II. and III. class

All relocations and modifications of the roads of II. and III. classes are proposed in the category C 7,5/60 with required parameters on spatial arrangements and perspective rations of the road. Total scope of relocations and modifications of the roads of II. and III. class of the version A red is 5,790 m, version B blue is 5,375 m and version C green is 6,720 m.

Bridge objects

Proposal of bridge objects is realized according to the standard STN 73 6201. Objects are proposed according to the standard STN 73 6203 for the Load Class A. Bridges on country roads are proposed for the Load Class B. The superstructure of bridge objects is proposed in accordance with approved bases of beam elements for superstructures. Monolithic objects are proposed according to generally valid principles for designing of ferroconcrete constructions.

Retaining and revetment walls

By reason of reducing of volume of earthworks, as well as reducing of permanently taken areas and ensuring of stability of slopes excavations and embankments, retaining and revetment walls are proposed. Retaining walls are mainly proposed as gabion spill-through constructions with max. height of 7.0 m above the terrain. Revetment walls are proposed as gravitation concrete constructions, where fair-faced surface will be tiled by quarry stone, or possibly, stability of the place with tendencies to land-slide will be secured by revetment piles walls made of large-radius piles anchored by earth cable anchors.

Synoptic table of main indicators of proposed routes of the road relocation

| Indicator | unit of measure | Relocation of the road I/18 |  |  | Relocation of the road I/74 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Version A red I/18 | Version B blue I/18 | Version C green I/18 | Version A red I/74 | Version B blue I/74 |
| Total length of the route | km | 48.260 | 46.365 | 49.255 | 56.665 | 57.580 |
| Bridges to 50 m | m2 | 3,662 | 3,377 | 1,652 | 9,326 | 7,099 |
| Bridges from 50 to 100 m | m2 | 11,158 | 11,480 | 8,302 | 7,158 | 4,002 |
| Bridges above 100 m | m2 | 43,316 | 68,743 | 67,960 | 64,539 | 57,687 |
| Other bridges over the road | m2 | 5,096 | 2,475 | 3,917 | 340 | 956 |
| Revetment and retaining walls | m2 | 4,400 | 9,600 | 19,700 | 4,400 | 4,400 |
| Modification of water courses | m | 1,775 | 3,640 | 3,670 | 1,560 | 1,740 |
| Drainage | m | 6,700 | 8,200 | 8,200 | 6,650 | 6,150 |
| Separators of oil products | pcs. | 10 | 16 | 14 | 12 | 10 |
| Waterlines | m | 1,420 | 1,420 | 1,400 | 1,880 | 2,090 |
| Line VN-22 kV | m | 16,500 | 13,500 | 13,500 | 16,400 | 11,900 |
| LV distributors | m | 1,200 | 800 | 840 | 3,200 | 3,930 |
| Gas pipelines | m | 1,000 | 1,730 | 1,440 | 1,680 | 2,130 |
| Road information system | m | 35,000 | 35,000 | 35,000 | 35,000 | 35,000 |
| Modification of signaling and safety lines | m | 26,600 | 15,350 | 15,400 | 11,950 | 8,410 |
| Noise barriers | m2 | 24,615 | 23,424 | 25,800 | 23,532 | 26,040 |
| Fencing | m | 2,490 | 2,540 | 2,540 | 2,000 | 2,000 |

II.7. 	Substantiating of necessity of proposed activity in respective locality (its positives and negatives)

Communications I/18 and I/74 connect the largest district cities of Prešov self-governing region with the regional capital Prešov. Existing communications have been in original cross-sectional profile as two-lane communications of varying cross-sectional profile for many years and increase of traffic has not been reflected in width modifications. Their overloading, also because of local traffic, causes problem with traffic, mainly in residential areas of the cities Vranov nad Topľou, Humenné and Snina.

Construction of capacity communication outside of built-in areas of respective villages will help to decrease the traffic on existing communications, which will have adequate capacity for remaining local traffic.

Collision free, safe, fluent alignment of transport with sufficient capacity requires construction of capacitive road of the Class C 11,5/100 will be in the offing constructed as R 11,5/100 in the section Lipníky - Vranov nad Topľou T 24,5/100, including interchange connections on existing communication network and service equipments of the communication.

II.8. 	Total costs:

## Version A red   I/18   :	574,552,589,- €

## Version B blue   I/18     :            			553,895,091,- €

## Version C green    I/18     :            			559,966,894,- €

Version A red I/74    :					606,940,601,- €

Version B blue    I/74     :				621,474,434,- €

Version A red I/18 + I/74	:			1,181,493,190,- €

Version B blue I/18 + I/74	:			1,175,369,525,- €

II.9. 	Statement on presupposed impacts of proposed activity overlapping the state border

With respect to location, scope and character of the activity, we can expect impacts caused by increased traffic load on Ukrainian side. Moreover, positive impacts of proposed activity on international transit and automobile transport are expected.

III. BASIC DATA ABOUT PRESUPPOSED IMPACTS OF PROPOSED ACTIVITY ON THE ENVIRONMENT, INCLUDING HEALTH, AND APPROXIMATION ABOUT POSSIBILITIES OF MEASURES ON THEIR MITIGATION

III.1. Impacts on population

III.1.1. Air pollution and noise from traffic

Air pollution caused by operation on relocation of roads I/18 and I/74 is explained in Emission report (DOPRAVOPROJEKT, a.s., 2011). To set the level of air pollution caused by automobile transport in respective area, mathematical modelling of scattering of pollutants from automobile sources was used. Besides the automobile transport on respective roads sections, the calculations did not reflect other sources, it means remote transmission and other stationary and mobile sources in the region and locality. According to the calculations, for average year concentrations, contribution of considered air pollutants to air pollution, that where caused by presupposed transport loading with respect to relevant emission limit is minimal in all solution versions and will not exceed permitted limit values.

Examination of noise caused by realization of proposed activity is explained in Noise report (DOPRAVOPROJEKT, a.s., 2011). Determining line source of noise in the area being solved is existing road I/18 or I/74. In this locality, hygienic limits of noise levels during day and night hours are exceeded. Therefore, construction - technological measures in order to decrease noise level were proposed. On the basis of presupposed calculation model, after construction of noise barriers, permitted limits of noise will be not exceeded.

III.1.2. Disturbance of comfort and quality of life of population

Construction activities will disturb the inhabitants by noise and increased dustiness mainly at the beginning of construction during earth works and preparation of the area for construction. Noise caused by transit of heavy-duty trucks will remain during the whole period of construction. This fact will be evident mainly during demolition works in connection with liquidation of objects that are situated in the route of solution versions. In case of version B blue of relocation of the road I/18 in the locality, where this road crosses the city Hanušovce nad Topľou in parallel with the railway through the valley of the stream, it is necessary to demolish 3 houses and one farm building in order to build bridge object over the valley.

It is supposed that the construction works will last more years and will be realized gradually in accordance with separate phases of the construction in different localities.

III.1.3. Social and economical consequences and dependences

Social-economical impacts of prepared construction will be evident after construction realization as a consequence of higher technological level of proposal against present state (modernization of technical conditions of the communications).

In case of users, social effects will be evident mainly by increasing of their safety and vehicle comfort and in case of people living near the road by decreasing of negative impacts of traffic on their environment. Another positives are fuel saving, decrease of operational costs and time spent by travelling, as well as improving of personal transport. Positive impact of investment realization is also its better servicing and creation of conditions for development of attractive locality and so establishment of job opportunities as a result of construction activity.

III.2. Impact of environment

III.2.1. Impacts on rock environment

All solution versions passes very variable geological environment. Within engineering-geological survey (DOPRAVOPROJEJKT, a.s., 2010), any technical survey works were not executed. The report was elaborated only on the basis of project documentation and available archive materials.

### In case of relocation of the road I/18 Lipníky – Strážske, the sections from the beginning of the road up to the village Bystré and from the village Nižný Hrabovec up to the city Strážske are the most difficult parts in all solution versions. On the contrary, the easiest section of the road is the section between the villages Čierne nad Topľou and Nižný Hrabovec, where the routes of all versions are mostly led in the bottom land of the river Topľa.

### In case of the road I/74 Strážske - Ubľa, in both two versions, the most difficult section is the last 20 km up to the state border.

III.2.3. Impacts on surface and underground water

In terms of possible impact on surface waters, the critical places are crossings of water courses, their modifications and relocations. Interference to meander of the river Topľa is of the greatest scope, because the relocation of the road crosses this river several times, as far as it is situated in presupposed flooding area of the river. Location of the road as well as proposal of bridge objects with greatest bridge opening should minimize the risk of increased floods caused by locating the road relocation into this flooding area of the river.

Endangering and vulnerability of underground water is similarly, as in case of surface water, connected with the sections of crossings, or approximation of communication to surface water courses. Temporarily, potential risk are also plant depots and equipments used during construction (possible leakage of soil water and contaminants into underground water).

During the operation of communication, in consequence of low flow and consequent insufficient dilution of waters, partly increased contamination of water courses, mainly by chlorides from road salts used during winter, can appear. More cumulative character of contamination by chlorides can be evident in covering sediments in the immediate vicinity of the road, where flush waters from the road will penetrate.

More serious contamination, or declining of quality of waters can appear in case of accidents, mainly of tanks transporting substances harmful for waters - in case of quick penetration of contaminants into the waters, e.g. pouring off directly to the stream. Certain measures can minimize negative impact on surface and indirectly also on underground waters (limitation of spreading of road salt in critical places, installation of safety fences, adjusting of slopes, etc.).

## III.2.4. Impacts on soil

Immediate impact of construction of communication on soil is its permanent occupation by the body of object. Occupations of soil are increased by temporary occupations needed for plant depot, manipulation tracks, dumping of soils. After storing and enriching with missing nutritive, denuded humus layer can be used for recultivation of areas of temporary occupations.

Under conditions of non-standard operation of the road, it means in case of more serious accident of motor vehicles connected with leakage of fuel, the surrounding soil can be polluted by oils with the possibility of their penetration to underground water or surface water courses.

With respect to better traffic and technological parameters of new communication, it is possible to expect decrease of risk of possible accidents connected with endangering of soil quality.

## III.2.5. Impact of biotopes

Communication mainly passes the locality, which is in 1. level of protection in the scope of provisions of § 12 act NR SR no. 543/2002 Coll. on protection of nature and landscape.

Proposed solution versions of relocation of the roads I/18 and I/74 passes through legislative protected areas (proposed locality of European importance SKUEV0063 Ublianka and protected bird area SKCHVÚ011 Laborecká vrchovina (Laborec mountains)). Their location requires liquidation of forest greenery and plants and greenery outside the forests of different scope, as well as interference into forest, banks biotopes or riparian stands of river courses.

With respect to impact on animals living in respective areas, the construction phase, as well as the phase of communication operation have negative impact on it. The most serious direct impact is liquidation of ecosystem or its part, when animate organisms are liquidated, as well as conditions necessary for their life. In this case, these changes are irreversible and given ecosystems will be lost in this locality forever. Side effect of construction is noise, vibrations, increased amount of air pollutants in this locality, where these effects did not appear before (in case of constructing new section of the road). This causes stress to animals and consequent change of their behavior. During large construction works, ecosystem or its parts are often mechanically damaged. Regeneration of damaged parts is possible, but it depends on degree of damage.

Fragmentation caused mostly by line constructions also presents an important influence on ecosystems. Proposed road divides coherent systems on two or more parts with barrier influence of construction. Negative impact of fragmentation is most significantly evident in case of ecosystems of water courses and ecotone communities on borders of coherent forest crops and other country, as far as these usually serve as biocorridors in the country and they are places of increased movement - migration of organisms. In case these biocorridors are disconnected by road communication, there are many crashes among animals and vehicles in these places.

III.3. Measures for reduction of negative impacts of the activity

In respective phase of project preparation of construction, it will be necessary to respect the following measures during preparation, realization and operation of the road:

III.3.1	Organizational measures

Emergency plan for construction is a part of organization of site construction of the contractor. This plan solves elimination of negative impact of the site on environment (dustiness, leakage of harmful pollutants, technical condition of the vehicles used during the construction, parking spaces, communications, and stores of fuel, transport lines and others). Terms of the plan will be elaborated in accordance to valid legislation.

During the construction, it is necessary to take measures for ensuring fluent and safe traffic by respective road signs (speed limitations, limitation of access, traffic diversions, etc.) on reserved communications.

III.3.2.		Measures for protection of inhabitants against unfavorable impacts of polluted air

During the construction, the air can contain increased concentration of gases and air pollutants from vehicles and construction mechanisms, as well as dustiness around the site caused by transit of mechanisms and manipulation with excavated material. In order to decrease concentration of harmful substances in the air, it is necessary to use only such mechanisms in case of which emissions are up to limits given by valid legislative regulations. By using of appropriate technical-organizational measures, it is possible to reduce negative impacts of above mentioned effects on environmentally acceptable level.

III.3.3.	Measures for elimination of unfavorable impacts of noise

During the phase of construction of designed activity, it will be not possible to protect the inhabitants against unfavorable noise caused by transit of construction mechanisms or by activities connected with construction procedures, mainly in immediate surrounding of roads used for transit of materials. By proper organization of works on the site, ensuring of failure-free condition of used machines and mechanisms, as well as not working during night hours, it is possible to reduce the impact of noise on endurable level that can be tolerated during the period of construction.

In the phase of operation of road relocation, the following noise protection measures will be proposed according to the conclusions of Noise report (Dopravoprojekt a.s., 2010):

- in the section of the road I/18 Lipníky – Strážske, total length of noise barriers will be: in case of version “A” red - 8,205 m with the area of 24,615 m2, in case of version “B” blue 7,808 m with the area of 23,424 m2 and in case of version “C” green 8,600 m with the area of 25,800 m2.

- in the section of relocation of the road I/74 Strážske – Ubľa total length of noise barriers will be: in case of version “A” red 7,844 m with the area of 23,532 m2 and in case of version “B” blue 8,680 m with the area of 26,040 m2.

III.3.4.	 Measures for protection of rock environment

In connection to rock environment, measures can be proposed only on general level. As far as from geological point of view, this is very complicated locality with several slide and potentially slide areas together with unsuitable composition of sub-base, concrete techniques of reconstructions require knowledge of locality with results from detailed geological survey that will be solved in further phases of project documentation.

During design preparation of the construction, by adjustment of grade line and optimalization of alignment of communications, it is possible to minimize interferences to unstable environment, necessity of cuts and fills with respect to aligned material balance, proposal of slope gradient, their adjustment, dewatering and proposal of retaining walls.

One possibility of rock environment protection is maximum utilization of excavated soil for construction of new objects. It is supposed that soil excavated during earth works will be incorporated in the body of the road. In case that excavated soil will be not appropriate for further construction purposes, it will be adjusted by special techniques in order to use it as much as possible. In case of materials from other sources, it is necessary to use existing bearings with materials in maximum and to open new bearings only if necessary.

III.3.5.	 Measures for protection of surface and underground waters

In order to minimize negative impact on surface or underground water during realization of proposed activity, it is important to eliminate or at least partially reduce impacts connected with construction itself:

- to execute earth works only to such extent that quality of underground water and water relations will be not disturbed, or they will be disturbed only in required range, or to execute all works during the period of low water levels,

- during the construction, to ensure respecting of safety regulations and technical standards while manipulating with oil products and to check technical condition of mechanisms and vehicles regularly,

- not to establish plant depots in areas, where more permeate rock layers are directly on the surface or just immediately under the surface,

- to equip the plant depots and mechanisms with protective means and with sufficient amount of absorption materials that can be used in case of accidents or leak of dangerous substances into the environment,

- during selection of routes of the road relocation versions, which pass through II. level of protected areas of water courses, it is necessary to have permission of respective water management body for realization of geological and other earth works.

Road drainage is designed in the sections, where the road relocation passes protected area of water courses. For protection against contamination of water sources, road drainage is designed in order to collect rainfall water from the surface of road and possible leaked polluting oil substances are collected in separators of oil substances with sedimentation part. After defecation, rainfall water is then led to water courses. Road drainage is proposed in relocation of the road I/18 in the section - km 9.000 - 10.000, in km 11.000 - 12.000 and in km 18.000 - 19.000.

In case of the road I/74, road drainage is proposed in the section in km 4.000 – 6.000, in km 17.000 – 19.000 and in km 34.000 – 36.000.

III.3.6.	 Measures for protection of soil

Negative impact on soil is mainly on permanent occupation of the soil, which will be minimized within technical solution of the construction.

During the construction, the measures have to pay attention on elimination or at least partially reduction of impacts connected with the construction itself:

- concretion of the soil during construction is reversible process and it can be eliminated by using mechanical recultivation in form of deep soil loosening,

- in case of intoxication and degradation of soil, it is necessary to stop using it and realize biological recultivation,

- in case of soil degradation, it is necessary to realize biological recultivation of respective soil fund after finishing of construction.

III.3.7.	 Measures for protection of biotopes

During the phases of construction and operation, the measures for protection of biotopes will be focused as following:

- during construction, to reduce deforestation only on necessary level, to protect other wood species near the site against possible mechanical damage,

- cutting down of non-forest bushes and tree verdure will be realized only out of nesting period,

- to minimize impacts to banks biotopes, to avoid devastation of banks of the rivers,

- to avoid interference to the bottom and banks of the river during fixing of bridge pillars,

- to leave free space between the bridge object and bank of the river in order to enable transition of animals,

- to ensure minimum clearance height – 2.60 m in case of smaller water courses,

- to design bridge objects (mainly above larger rivers) in such a way, so they do not create a barrier in the migration of birds,

- to situate plant depots, parks for technical equipments and other additional construction objects in the area with poor species diversity and to reduce transit of construction mechanisms only within the site,

- to execute recultivations and landscape gardening of the localities disrupted by construction, to reconstruct disrupted bank vegetation after completion of construction works,

- in further degree of project documentation, to realize inventory control and social assessment of woody species that will be necessary to liquidate, and to realize landscape gardening of the areas designated by respective landscape protection authority up to the amount of calculated social value,

- to realize vegetation adjustments of communication slopes - cuts and fills and internal areas of crossroads.

III.3.8.	 Measures for protection of landscape, incorporation of the work into the landscape

Best measures for improvement of aesthetic effect of longitudinal and vertical alignment of construction and for incorporation of technical work into the landscape are vegetation adjustments on slopes of communication. These adjustments also helps to strengthen non-forest tree and bush vegetation in agricultural locality. Selection of composition of species of trees and bushes will be oriented on original typical species of respective area. In terms of aesthetic perception of the construction by inhabitants, it is necessary to suggest appropriate architectural solutions of individual construction objects, bridges, crossroads and others. Still another step, which can help to incorporate the communication and crossroads into the country is recultivation of damaged localities, what will form appropriate conditions for further recultivation, it means restoring of biotic components of the landscape not only from physical point of view, but also from functional one.

III.4.	Monitoring of construction impacts on environment

The aim is to monitor impacts of construction and operation of the road relocation on separate parts of environment, as well as to verify functionality of proposed measures. On the basis of defined impacts and extent of their impact on environment, we suggest monitoring of noise, monitoring of surface and ground water, monitoring of chosen biotopes and groups of animals, monitoring of rocks and geotechnical monitoring.

IV. COMPARISON OF VERSIONS OF PROPOSED ACTIVITY

AND PROPOSAL OF OPTIMUM VERSION

Main aim of this investment is to build capacitive communication outside of built-in area of respective cities and villages in order to increase safety of transport, traffic speed, operating efficiency of crossroads, and so to decrease accident rate and consumption of fuel, and also to increase quality of the environment in residential areas of the cities and villages by decreasing of production of air pollutants and noise level.

The process of evaluation of impact of relocation of the roads I/18 and I/74 on inhabitants, socio-economic sphere and environment also tries to identify such version solutions, which will mean the lowest and considerate impact on all parts of the environment. On one hand, there are impacts that will be negative during the phase of construction or operation and on the other hand there are positives, which the construction will bring from the perspective of long term prognosis, mainly towards the inhabitants and traffic. All reviewed versions have their advantages, but also disadvantages.

Assessment report recommends that the optimal solution is a combination of several versions of designed sections listed below:

In section I/18 Lipníky – Vranov (MÚK Vranov north) 		- version red

In section I/18 Vranov (north) – Vranov (south) 		- version red

In section I/18 Vranov (south) – Strážske south  		- version blue

In section I/74 Strážske south – Humenné   			- version red

In section I/74 Humenné – Snina   				- version blue

In section I/74 Snina – Kolonica				- version red

In section I/74 Kolonica – Ubľa					- version red

V. acknowledgement of data accuracy

On behalf of activity compiler:

In Bratislava, 30. June, 2011

Ing. Ján Longa

Chief solutionist

DOPRAVOPROJEKT, a.s. Bratislava

Authorized representative of activity compiler

On behalf of proposer:

In Bratislava, 30. June, 2011

Ing. Jozef Fabian

Director of  IV and SC Košice

The Slovak Road Administration, IV and SC Košice